| ### Company: |  |  |  |  |  | Alexander Carpenter 53 JOALAH acres, Ferny Hills 4055 ABN:39703159407 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Complete if specific Client / Contract requirement OR Refer to WHSMP / PMP for project specific details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SWMS Approval |  |  | This SWMS has been developed to suit our operations, in accordance with relevant legislative requirements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Client: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project Address: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |  |  |
| Scope of Works: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Alex Carpenter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |
| Client Contact: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Approved: |  |  |  |  |  |  | 04/24 |  |  |  |  |  | Revision Due: |  |  |  |  |  |  | 04/25 |  |  |
| SWMS CONSULTATION / APPROVAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APPROVAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  | Position |  |  |  |  |  |  |  | Name |  |  |  |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  |  |  | By: |  |  |  |  |  |  | Alex Carpenter |  |  |  |  |  |  |  |
| Alex Carpenter |  |  |  |  |  |  | Project Manager |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Supervisor |  |  |  |  |  |  |  |  |  |  | Position: |  |  |  |  |  |  | Director |  |  |  |  |  |  |  |
| Jeff Rose |  |  |  |  |  |  | WHS Consultant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Builder |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 1/04/2024 |  |  |  |  |  |  |  |
| SWMS REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Activity: |  |  |  |  | General Work on a Construction Project (Including manual handling) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OHS Legislation |  |  |  |  | QLD WHS Act and Regulations 2011 QLD Electrical Safety Act 2002 and QLD Electrical Safety Regulation 2013 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High Risk Activity: |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Code of Practice: AS Standards: |  |  |  |  | COP – How to manage WHS risks - 2021 COP – Hazardous manual tasks – 2021 COP – Managing noise and preventing hearing loss at work – 2021 COP - Managing-risks-falls-workplaces - 2021 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COP – Managing electrical risks in the workplace – 2021 COP – Managing risks of hazardous chemicals in the workplace – 2021 COP – WHS Consultation, co-operation & co-ordination – 2021 COP – Managing the work environment and facilities - 2021 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Referenced SWMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Competency |  |  |  |  | White National Induction Card, QLD Blue Card 30215 or equivalent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant & Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PPE Required = Mandatory ? = As required ( ) = Not required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Sun hat |  |  |  |  | Hearing |  |  | Eye |  |  | Hard hat |  |  |  | Gloves |  |  |  | Dust mask |  |  |  | Enclosed shoes |  |  |  |  | High Vis |  |  |  |  | Face shield |  |  |  |  |  |  |  | Safety Harness |  |  |  | Weld Shield |  |
|  |  |  |  |  | ? |  |  |  |  | ? |  |  | ? |  |  | ? |  |  |  | ? |  |  |  | ? |  |  |  |  |  |  |  |  |  |  |  |  |  | ? |  |  |  |  |  |  |  | ? |  |  |  | ? |  |
| General Hazard Profile: |  |  |  |  | Noise > 85dBA ☒ |  |  |  |  |  |  |  |  | Dust / Fumes ☒ |  |  |  |  |  |  | Manual Handling > 20kg ☒ |  |  |  |  |  |  |  |  |  | Hazardous Substances ☒ |  |  |  |  |  |  |  |  |  |  |  |  |  | Machinery / Plant ☒ |  |  |  |  |  |  |
|  |  |  |  |  | Vibration ☒ |  |  |  |  |  |  |  |  | Live Traffic ☒ |  |  |  |  |  |  | Repetition > 2 hrs / shift ☒ |  |  |  |  |  |  |  |  |  | Environment / Space ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  | Electricity ☒ |  |  |  |  |  |  |
| Daily Prestart: |  |  |  |  | Complete daily visual inspection of all tools. Complete daily prestart of all plant. Check material required is on site. Complete SLAM and/or JSEA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RISK AND CONTROL MATRIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Determine which hazards apply for the plant item. - Perform a risk assessment for each hazard identified by determining the consequence and likelihood for each hazard in the Qualitative Risk Assessment Matrix to obtain a risk score and risk level. Determine appropriate controls and rescore the hazard. Determine if the residual risk is acceptable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequence Table / SCORE RATING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | likelihood Table / SCORE RATING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Given that the event occurs, what is the likely outcome? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | How likely is it that the event will occur? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Score | DESCRIPTOR | CONSEQUENCE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Score |  |  | DESCRIPTOR |  |  |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SAFETY |  |  |  |  |  |  |  | ENVIRONMENTAL |  |  |  |  |  |  | ASSETS |  |  |  | REPUTATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Insignificant | Minor incident. No treatment |  |  |  |  |  |  |  | Negligible impact |  |  |  |  |  |  | Nil damage |  |  |  | Nil |  |  |  |  |  |  |  |  |  | 1 |  |  | Rare |  |  |  |  | May occur, in exceptional circumstances |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Low | Minor injury / first aid |  |  |  |  |  |  |  | Minor, local, no long-term effects |  |  |  |  |  |  | Minor damage / no impact on operations |  |  |  | Local issues |  |  |  |  |  |  |  |  |  | 2 |  |  | Unlikely |  |  |  |  | Could occur at some stage though is improbable |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Medium | Minor Medical treatment req’d |  |  |  |  |  |  |  | Mod impact contained locally |  |  |  |  |  |  | Delays / costs. Impact on operations |  |  |  | Isolated bad publicity |  |  |  |  |  |  |  |  |  | 3 |  |  | Possible |  |  |  |  | Event might occur at some time |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Major | Serious injuries, hospitalization |  |  |  |  |  |  |  | High impact, widespread impacts |  |  |  |  |  |  | Serious damage long delays / costs high |  |  |  | Impact on client / community relations |  |  |  |  |  |  |  |  |  | 4 |  |  | Likely |  |  |  |  | Event will probably occur in most circumstances |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Severe | Fatalities |  |  |  |  |  |  |  | Major damage, long term impact |  |  |  |  |  |  | Plant write off / high repair costs & impacts |  |  |  | Loss of Contracts / long term impacts |  |  |  |  |  |  |  |  |  | 5 |  |  | Almost Certain |  |  |  |  | Event expected to occur in most circumstances |  |  |  |  |  |  |  |  |  |  |  |
| qualitative risk assessment matrix |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Risk Level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HIERARCHY of CONTROLS |  |  |  |  |  |  |  |  |  |  |
| Likelihood |  |  | CONSEQUENCE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Risk Rating |  |  |  |  |  |  |  |  | Score |  |  |  |  |  |  | 1 |  |  |  |  |  | ELIMINATION |  |  |  |  |
|  |  |  | Level 1 |  |  |  |  | Level 2 |  |  | Level 3 |  |  |  |  | Level 4 |  | Level 5 |  |  |  |  |  | Extreme |  |  |  |  |  |  |  |  | 15 - 25 |  |  |  |  |  |  | 2 |  |  |  |  |  | SUBSTITUTION |  |  |  |  |
|  |  |  | Insignificant |  |  |  |  | Low |  |  | Medium |  |  |  |  | Major |  | Severe |  |  |  |  |  | High |  |  |  |  |  |  |  |  | 10 - 14 |  |  |  |  |  |  | 3 |  |  |  |  |  | ENGINEERING |  |  |  |  |
| 5 – Almost Certain |  |  | M-5 |  |  |  |  | H-10 |  |  | E-15 |  |  |  |  | E-20 |  | E-25 |  |  |  |  |  | Moderate |  |  |  |  |  |  |  |  | 5 - 9 |  |  |  |  |  |  | 4 |  |  |  |  |  | ADMINISTRATIVE |  |  |  |  |
| 4 – Likely |  |  | L4 |  |  |  |  | M-8 |  |  | H-12 |  |  |  |  | E-16 |  | E-20 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  | 1 - 4 |  |  |  |  |  |  | 5 |  |  |  |  |  | P.P.E |  |  |  |  |
| 3 – Possible |  |  | L-3 |  |  |  |  | M-6 |  |  | M-9 |  |  |  |  | H-12 |  | E-15 |  |  |  |  |  | - The risk levels require different time frames for action. Extreme risks require immediate action, low risk may not need any actions. - Use the Hierarchy of controls to reduce the residual risk to as low as possible. See table below for additional details. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 – Unlikely |  |  | L-2 |  |  |  |  | L-4 |  |  | M-6 |  |  |  |  | M-8 |  | H-10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 - Rare |  |  | L-1 |  |  |  |  | L-2 |  |  | L-3 |  |  |  |  | L-4 |  | M-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hierarchy of Controls - DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Elimination |  |  |  |  |  |  | Controls the risk by eliminating the hazard e.g. positioning controls at ground level eliminates risk of fall from heights. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Substitution |  |  |  |  |  |  | Replace the hazard (e.g. plant or substance) with another that has a lower and / or zero risk. This may also eliminate the risk. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engineering / Isolation |  |  |  |  |  |  | Remove or separate people from the source of the hazard e.g. guarding, noise barriers, install welding shields etc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Administrative measures |  |  |  |  |  |  | Use policies, procedures, signs, staff rotation and training to minimise the effects of the risk. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment (PPE) |  |  |  |  |  |  | Provide equipment or clothing designed to protect the worker e.g. ear muffs, safety glasses, gloves, steel capped boots. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 - 4 LOW RISK 5 – 9 MODERATE RISK 10 – 14 HIGH RISK 15 – 25 EXTREME RISK |  |  |  |  |  |  |  |  |  |
| HAZARD IDENTIFICATION | POTENTIAL RISKS | Risk Rating |  |  | CONTROL METHODS Score as per Risk Matrix on previous page. | Residual Risk |  |  | MONITOR & REVIEW |
|  |  | L | C | T |  | L | C | T |  |
| General hazards in construction to both employees and other people. | Serious damage/injury due to lack of awareness of potential hazards. | 4 | 4 | 16 | - 30215 Course in General Safety Induction (Construction Industry) or equivalent - General Safety Induction - Specific site safety inductions at the commencement of each job, conducted by PC. - Staff are to be trained / assessed as competent in their roles. - Be aware of and follow all safety signs / instructions. - All people not fully site inducted are to report to the supervisor upon arrival at the job site. Please stop & direct all people to the Supervisor. - REMEMBER If you are not sure about something or you see something that you feel is unsafe, STOP / ASK / INFORM your Supervisor. | 2 | 4 | 8 | Supervisor All Staff |
| Use of portable electrical equipment | Electrocution | 4 | 5 | 20 | - All 240-volt electrical equipment will be tested and tagged up to date and be included on the electrical register. - All electrical tools to be connected to RCD’s. Test RCD’s daily - All electrical equipment to be in good condition. - All persons using electrical equipment to be competent to fulfill the task asked of them. - Ensure electric leads are plugged into power boards, fed up through the bottom hole and that the front door is closed. - Do not use extension cords that exceed 30m. Do not connect extension leads to each other. | 1 | 5 | 5 | Supervisor All Staff |

| Use of chemicals / hazardous substances. | Damage to the environment / general control requirements | 4 | 4 | 16 | - Storage and handling of hazardous goods to comply with the standards to ensure chemicals are not poured into water systems. - Unused, unwanted or waste products shall not be stored / disposed of on site. Ensure they are disposed of correctly. - Spillage and contamination shall be immediately contained. - SDS available for all chemicals and staff trained in their use. - Risk assessments completed for all hazardous substances. - Substitute chemicals with less harmful ones wherever possible. | 1 | 4 | 4 | Supervisor All staff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inhalation / Ingestion Nausea / death | 4 | 4 | 16 | - Respiratory protection to be worn, if required by SDS - Use in well ventilated room where ever possible. | 2 | 4 | 8 | Supervisor All staff |
|  | Skin contact Burns or serious injury | 3 | 3 | 9 | - Protective clothing to be worn in accordance with SDS - Authorised personnel only in work area - Water / soap available to wash off chemicals. | 2 | 3 | 6 | Supervisor All staff |
|  | Eye Contact Burns / loss of sight | 3 | 4 | 12 | - Have eye wash station available / running water where possible - Use of goggles as per SDS. | 2 | 4 | 8 | Supervisor All staff |
| Work within A client’s workplace. This applies where the client and / or staff wish to visit the site during the construction phrase. | Injury to Clients staff and or visitors | 3 | 4 | 12 | - Establish with client accepted workspaces limits of the work site. - Ensure the construction site is sufficiently fenced / barricaded to minimise the likelihood of unauthorized people entering the site. - If applicable, make arrangements for the client to have set times / protocols to visit the site during construction. E.g. short periods during lunch breaks for guided tours of the works and progress under the supervision of our personnel. - Establish requirements e.g. enclosed footing, safety vest for all visitors. - Maintain communications with client as access to site will vary depending on conditions / activities. | 2 | 4 | 8 | Supervisor, All staff, Client, client’s staff |
| Work in public areas |  | 3 | 3 | 9 | - Establish with client workspaces limits of the construction area. - Ensure the construction site is sufficiently fenced / barricaded to minimise the likelihood of unauthorised people entering the site. - If necessary, establish set construction entry points that are controlled by full time employees to ensure only inducted / approved persons are allowed access. | 2 | 3 | 6 | Supervisor All staff |

| Facilities | - Lack of basic amenities including drinking water, toilets, lunch facilities, clean washing water | 3 | 4 | 12 | - Ensure toilet facilities are available onsite and / or nearby. Use portaloos onsite if no reasonable access to toilets. - Drinking water to be available to all persons on site. (Water bottles) - Clean water for washing to be made available. - Site office / lunch room to be made available on major projects and / or where appropriate. Where possible utilise nearby park facilities etc for lunch breaks. | 1 | 4 | 4 | Supervisor Plant operator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work in / over / adjacent to water. Whilst there is no need to enter water during operations there is a slight risk that plant may topple into the water and/or persons may fall into water. | - Risk of drowning - Damage to plant - Environmental impacts if plant | 2 | 5 | 10 | - When operating near water take extra precautions to ensure that the plant is not too close to the edge and is at risk of toppling into the water. - Establish limits / boundaries. If necessary, use bollards and / or other visual aids to establish boundaries. - Where practical use physical barriers e.g. scaffolding to minimise chance of falling into water. If there is still a risk of operators / staff falling into water take the following: - Ensure all persons can swim to a reasonable level. - Have a life vest available. - A qualified First Aider available - Ensure there are standby persons available. | 1 | 5 | 5 | Supervisor Plant operator |
| Damage to feet by falling objects. | - Broken bones or loss of limbs. | 3 | 4 | 12 | - Foot protection to be worn at all times, whilst on construction. Whilst steel cap footwear is encouraged as a minimum footwear needs to be fully enclosed with suitable sole for grip. | 2 | 4 | 8 | Supervisor All Staff |
| Damage to eyes by dust, flying objects. Exposure to ultra violet rays. | - Corneal damage, cataracts and loss of eyesight. | 3 | 4 | 12 | - Eye protection to be worn if there is a risk of flying objects. - Good practice to wear glasses to minimize damage by dust etc. - Eye protection to worn when working in sunlight with 100% UV protection. | 2 | 4 | 8 | Supervisor All Staff |

| Struck on head by falling objects. | Serious injury or death. | 3 | 5 | 15 | - Head protection to be worn where there is a risk of being struck on the head by a falling object and / or it’s a site requirement. Look for signs. | 1 | 5 | 5 | Supervisor All Staff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Exposure to noise | Noise induced hearing loss | 3 | 4 | 12 | - Ensure hearing protection is used when using tools etc that make a loud noise. Ensure PPE is suitable for the task / noise level. - All persons to be trained in use of PPE. - Where required do not start noisy operations early in morning / late in afternoon / on weekends (Sunday). | 1 | 4 | 4 | Supervisor All Staff |
| Exposure to climate (Heat / humidity). | Heatstroke, exhaustion dehydration. | 3 | 4 | 12 | - Drink plenty of water. Be aware of early signs e.g. dizziness, exhaustion etc. - If possible rotate work / do heavy work in early morning / late afternoon to avoid heat during middle of day. | 1 | 4 | 4 | Supervisor All Staff |
| Skin exposed to ultra violet rays. | Skin cancer and mild burns. | 3 | 4 | 12 | - Slip, Slop Slap & Wrap. Long sleeve shirt & pants with hat & glasses best option. Otherwise use sunscreen. | 1 | 4 | 4 | Supervisor All Staff |
| Manual Handling | Back or muscular injury Bruising, scratches etc. | 3 | 5 | 15 | - Train all staff in correct lifting procedures. - Use mechanical means, wherever possible. - Try to use products etc that are easily lifted, e.g. lighter weights, less awkward to carry, etc. - Store objects between shoulder and thigh height. - Rotate employees on a regular basis - Plan your lift, especially with more than one person - Minimise distances objects need to be carried. - Avoid twisting movements when lifting. - Avoid holding the load away from your body. - Avoid jumping from heights, which may jar your back. - Avoid jarring movements or carrying live loads. | 1 | 5 | 5 | Supervisor All staff |

| Methods for Monitoring and reviewing the effectiveness of the chosen control measures |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Workers: | Follow the procedure and report any inconsistencies / problems. |  |  |  |  |  |  |  |  |  |  |
| Supervisor: | Ensure workers have all necessary tickets, PPE to perform their tasks safely prior to commencement of work. Ensure all electrical equipment is fit for use and RCD’s are tested. |  |  |  |  |  |  |  |  |  |  |
| Monitoring and Review of SWMS Use and Effectiveness Record |  |  |  |  |  |  |  |  |  |  |  |
| Observation Log: | No 1 |  | No 2 |  | No 3 |  | No 4 |  | No 5 | No 6 |  |
| Initial: |  |  |  |  |  |  |  |  |  |  |  |
| Date: | / / |  | / / |  | / / |  | / / |  | / / | / / |  |
| EMPLOYEE TRAINING |  |  |  |  |  |  |  |  |  |  |  |
| I acknowledge that I have been trained in this SWMS, the controls are clearly understood, my qualifications are current and I will comply with the SWMS. I understand that it is my duty and obligation to advise my supervisor, manager or the site supervisor if I consider any task given to me or asked of me is – for any reason – outside of my capabilities at that time – or hazardous, dangerous or potentially damaging to the environment. |  |  |  |  |  |  |  |  |  |  |  |
| NAME |  | SIGNATURE |  | DATE |  | NAME |  | SIGNATURE |  |  | DATE |
| Brian Patterson |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |
|  |  |  |  | / / |  |  |  |  |  |  | / / |